EXCAM Sampling/Processing Summary

| Sample Type | Collected (volume) | Sample Processing | E.coli Detection Method | E.coli Detection (volume) |
| --- | --- | --- | --- | --- |
| Drinking water | 1L | 20mL of the sample was aliquoted from the original sample collection bag and used for E.coli testing. | EC MUG | A total of 15.84ml sample was tested using a 96-deep well plate. (88 wells). 180ul of the sample was plated in each well with 1.62mL of EC MUG broth. |
| Bathing Water | 1L |  | EC MUG |  |
| Hand rinse (Mother, Sibling, Child) | 200mL |  | EC MUG |  |
| Areola Swab | 4mL | Swabs were resuspended into 5ml of peptone water. 1 mL of the sample was used for direct plating and enrichment for Campylobacter. The rest of the suspension was transferred into a 15 ml tube and centrifuged at 4,500 RPM for 10 min, and the pellet was resuspend in 3 ml of sterile peptone water. Finally, 1mL of concentrated sample is aliquoted for E.coli testing. | EC MUG | 1mL of the sample was tested using five wells in a 96-deep well plate. 180ul of sample was plated in each well with 1.62mL of EC MUG broth. |
| Breastmilk | Varies (0.5ml-5mL) | Depending on availability, 1mL was aliquoted for E.coli testing. | Chromocult | 200ul of sample/plate. Serial 1:10 fold dilutions performed in peptone water; up to 102-fold dilution was plated |
| Soil Surface | Boot socks (15mL) | The liquid extracted from the boot socks was collected in the 15 ml tube. 1mL is aliquoted for E.coli testing. | Chromocult |  |
| Fomites | Sponge (5mL) | Fomites were resuspended into 10 ml of peptone water. 1 mL of sample was used for direct plating and enrichment for Campylobacter. The rest of the suspension was transferred into a 15 ml tube and centrifuged at 4,500 RPM for 10 min and the pellet was resuspended in 3 ml of sterile peptone water. 1mL of concentrated sample is aliquoted for E.coli testing. | EC MUG | 1mL of the sample was tested using 5 wells in a 96-deep well plate. 180ul of sample was plated in each well with 1.62mL of EC MUG broth. |
| Food | Varies (1g-10g) | 1g of food into a 15 ml tube containing 9 ml of peptone water. 1 mL of sample was used for direct plating and enrichment for Campylobacter. The rest of the suspension was transferred into a 15 ml tube and centrifuged at 4,500 RPM for 10 min and the pellet was resuspended in 3 ml of sterile peptone water. 1mL of concentrated sample is aliquoted for E.coli testing. | EC MUG /Chromocult (Depends on food consistency) | 1mL of the sample was tested using 5 wells in a 96-deep well plate. 180ul of sample was plated in each well with 1.62mL of EC MUG broth. |